# Oscar Pizey - 20537268

# Robot Drawing Program – System Manual

# Outline of the Problem to be Solved

At the top-level, the program must take a font and a letter height, then draw a text onto a piece of paper with a robotic arm.

The font must be read into the program through a text file containing each character, its ASCII code, the number of instructions contained within it, and the subsequent instructions. The program must be able to fetch any character and list out its coordinate list.

The letter height must be specified by the user and fall within a range of 4-10mm. If the user enters a value outside of this range, they will be prompted to re-enter an acceptable value. The height must be stored as a float, since the user may wish to specify a non-integer value.

The program must then take a text file and decompose it into its constituent words, letters and whitespace. It must be able to catch any errors within this process and return them without crashing. Once sorted, the length of the word at the specified scale must be able to fit onto the canvas. If the length is higher than this, then an error message is returned, and the program should terminate. If within the required length, it should be compared against the space remaining in the line, offset from the space taken up by previous words plotted, which decides if it will be written on a new line or not.

Before beginning writing, an initial packet of GCODE should be passed to the robot to set it up ready for the writing instructions. The GCODE should be passed line by line through a serial port to the robot arm. The robot will accept one line at a time and will respond “ok” when ready for another. The program should only send another command once this “ok” is received.

Each word should then be written out letter by letter. For each letter, the appropriate font entry should be accessed, and the instructions extracted, specifying the coordinates for the pen to move to and whether to put the pen up or down. For every instruction, the X, Y and P coordinates should be passed through a GCODE generator to convert them into a string of text. If the pen up/down state has changed from the previous instruction, then a P0/P1000 instruction should be passed too.

These GCODE instructions should be passed to the robot. After the final instruction, the pen should return to the up state, at 0,0 position.

# Project Files

main.c

generateGCODE.h

getDrawWord.h

loadFont.h

structs.h

SingleStrokeFont.txt

test.txt

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| letterHeight | float | The raw height of the letters in mm. Acceptable range between 4-10mm |
| letterScale | float | The height of the letters, scaled against the default height within the fontData file. |
| fontData | struct | Structure containing every letter within font file, each letter is a structure containing C, N, and all instructions. |
| drawWord | Int | The current word to be drawn |
| filePosition | Int | Position of the next word to be read out from the file. Increments by the word length each loop. |
| positionMarker | Int | Keeps track of current position within the file |
| X | Int | Variable used in FOR loops to iterate over each letter in the word within the file |
| remainingWidth | float | Width of canvas minus currently plotted letters |
| wordWidth | Float | The calculated width of the current word |
| xOffset | float | Offset along x for letters to ensure spacing between |
| yOffset | float | Controls newlines and spaces letters along the y direction |
| drawLetter | Int | Used to iterate over word within the FOR loop |
| penCommand | boolean | Tells the GCode compiler whether to append a pen instruction depending on whether it changed the previous step. |
| GCODELine | Char | Char array to store the GCODE output before being sent to the buffer |
| currentLine | Int | Iterate over each line of instructions within a letter |

# Function Declarations

int loadFont (struct fontData *letters)

Parameters:

fontData *letters – points to the nested structure-array to be read into

Return value – returns 0 for successful operation, 1 for error

void generateGCODE (float X, float Y, float P, float scale, float xOffset, float yOffset, int setPen, char *GCODE)

Parameters:

X, Y, P – float values of GCODE instruction for that line

scale – linear scaling factor for instructions

xOffset, yOffset – offsets for characters to control position within canvas

setPen – specifies whether to send a pen update instruction, based on whether it changes from previous instruction.

*GCODE – pointer back to char array which will hold the line of GCODE

int getDrawWord (int filePosition, int *drawWord, int *x)

Parameters:

filePosition – position at which to begin reading the file into memory

*drawWord – pointer to char array where current word to print is stored

*x – pointer to update x within loop

Return – filePosition returned after increment

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| Main() – Letter height input | Too low | 3 | Error – Retry Input |
|  | Lower bound | 4 | Accepted |
|  | Within range | 5 | Accepted |
|  | Upper Bound | 10 | Accepted |
|  | Too high | 11 | Error – Retry Input |
|  | Float | 5.5 | Accepted |
|  | Negative | -5 | Error – Retry Input |
|  | Symbol/Character | A@#/¦\ | Error – Retry Input |
| Main() – Input text | Empty text file |  | Nothing. |
|  | Unknown Characters | Characters not within font file | Error – Character not found |
|  | String too long to be drawn | Supercalifragilisticexpialidocious | Error – Word too long |
|  | String with too many newlines | \n\n\n\n\n\n\n\n | Error – Too many lines |
| getLetterWidth() | Character with 1 line of instructions. | “ “ | Error, n-1 falls out of range |
| sendSerial() | Connectivity | Disconnected serial port | Error – Check connection |

# Flowchart(s)

Included as separate PDF on Moodle and Github

# AI Statement

I have not used AI anywhere for anything within this coursework, and confirm that it is all my own work.